Department of Computer Science & Engineering

Microprocessor & Computer Architecture

MPCA-Laboratory/Assignment/Hands-on/Project

UE20CS252

NAME: VISHWAS M                                                                                 SEC : F

SRN : PES2UG20CS390                                                                             DATE: 22/02/2022

| Sl. No | Programs |
| --- | --- |
| Week No.5 | 1. Write a program in ARM7TDMI-ISA to generate Fibonacci Series and store them in an array. Program: Ldr R0,=A; Mov R10,#9 Mov R1,#0 Mov R2,#1 Str R1,[R0],#4 Add R3,R2,R1 Loop: Str R3,[R0],#4 Add R3,R2,R1 Mov R1,R2 Mov R2,R3 Sub R10,R10,#1 Cmp R10,#0 Beq Exit Bne Loop Exit: Swi 0x11 A: .word Screenshot: 2. Write a program in ARM7TDMI-ISA to find smallest number in an array of n 32 bit numbers Program: .text ldr r0,=data1 ldr r4,=0x05 ;//length of loop ldr r1,[r0],#04 sub r4,r4,#01 back: ldr r2,[r0] cmp r1,r2 bls less ;// branch on low mov r1,r2 less: add r0,r0,#04 sub r4,r4,#01 cmp r4,#00 bne back str r1,[r3] ;// smallest value stored in memory location stop: b stop .data data1: .word 10,20,7,8,100 Screenshot: 3. Write a program in ARM7TDMI-ISA to multiply 2 matrices of order3. i.e., implement c[i][j]=c[i][j] + a[i][j] x b[i][j]. a. Use MLA instruction b. Use MUL instruction Program: ; MULTIPLICATION OF 2 MATRICES. .DATA A: .WORD 1,2,3,4,5,6,7,8,9 B: .WORD 1,2,3,4,5,6,7,8,9 C: .WORD 0,0,0,0,0,0,0,0,0 .TEXT LDR R0,=A LDR R1,=B LDR R2,=C MOV R3,#0 ;INNER LOOP COUNT I INDEX MOV R4,#0 ;OUTER LOOP COUNT J INDEX MOV R10,#3 ; NUMBER OF ELEMENTS IN A ROW MOV R8,#0 ;VALUE OF K LOOP1:MLA R11,R3,R10,R8 MOV R11,R11,LSL #2 LDR R5,[R0,R11] MLA R12,R8,R10,R4 MOV R12,R12,LSL #2 LDR R6,[R1,R12] MUL R11,R5,R6 ; REGISTER R11 IS REUSED. ADD R9,R9,R11 ADD R8,R8,#1 ; INCREMENT K INNERMOST LOOP CMP R8,#3 BNE LOOP1 MLA R12,R3,R10,R4 ; STORE THE IN C[I][J] MOV R12,R12,LSL #2 STR R9,[R2,R12] MOV R8,#0 ; K=0 MOV R9,#0 ; C[I][J]=0 ADD R4,R4,#1 CMP R4,#3 BNE LOOP1 MOV R4,#0 ADD R3,R3,#1 CMP R3,#3 BNE LOOP1 SWI 0X011 .END Screenshot: 4. Write a program in ARM7TDMI-ISA to transfer a block of 256 words stored at memory location X to memory location Y using Load Multiple and Store Multiple instructions. The rate of transfer is 32 bytes. Programs: // Program to transfer a block of data from location X to location Y. .DATA A: .WORD 23,43 B: .WORD 0,0,0,0,0,0,0 .TEXT LDR R4, =A //INITIALIZATION OF THE BLOCK ADDRESSES LDR R5, =B loop: LDMIA R4!, {R0-R1} STMIA R5!, {R0-R1} CMP R11,#16 BNE loop Screenshot: Student exercises: 1. Write a program in ARM7TDMI-ISA to add 2 matrices of order3. i.e., Implement c[i][j]= a[i][j] + b[i][j]. Program: //TO FIND SUM OF N DATA ITEMS IN THE MEMORY.STORE THE RESULY //USING PRE-INDEXING ADDRESSING WITH WRITE BACK MODE .DATA A: .WORD 1,2,3,4 B: .WORD 1,2,3,4 SUM: .WORD 0,0,0,0 .TEXT MOV R2,#0 LDR R8, =A LDR R9,=B LDR R10, =SUM MOV R11,#1 SUB R8,R8,#4 SUB R9,R9,#4 SUB R10,R10,#4 MOV R12,#1 LOOP: LDR R6,[R8,#4]! LDR R7,[R9,#4]! ADD R2,R6,R7 ADD R11,R11,#1 STR R2,[R10,#4]! CMP R11,#5 BNE LOOP SWI 0X011 .END Screenshot: 2. Write a program in ARM7TDMI-ISA to find the ROWSUM of a matrix. Program: .DATA A: .WORD 1,2,3,4,5,6,7,8,9 SUM: .WORD 0,0,0 .TEXT MOV R2,#0 LDR R8, =A LDR R10, =SUM MOV R11,#1 SUB R8,R8,#4 SUB R10,R10,#4 LOOP: LDR R6,[R8,#4]! ADD R2,R2,R6 ADD R11,R11,#1 CMP R11,#4 BNE LOOP STR R2,[R10,#4]! MOV R2,#0 ADD R3,R3,#1 CMP R3,#3 SUB R11,R11,#3 BNE LOOP SWI 0X011 .END Screenshot: |