NAME: VISHWAS M

SRN: PES2UG20CS390

SEC:F

DATE:15/03/2022

WEEK:7

Department of Computer Science & Engineering

Microprocessor & Computer Architecture  - UE20CS252

| Sl. No | Programs |
| --- | --- |
| Week No.7 | 1. Demonstration of programs using plug-ins using ARMSIM. a. Set the LED to be light up. .text mov r0, #0 loop: swi 0x201 ; to light up LED , r0= 1 means right led light up, r0=2 means left ; LED and r0=3 means both LED light up add r0, r0, #1 mov r4, #64000 delay: sub r4, r4,#1 cmp r4, #0 bne delay cmp r0, #3 ble loop .end b. Display hexadecimal digits [0-9,A-F] on the 8 segment display. ; Program to display 0 to F and F-0 on the 8 segment display depending ; on the which black button is pressed .text .global _start begin: mov r0, #0 mov r2,#0 again: swi 0x202 ; check whether ; black button pressed or not cmp r0, #1 beq loop1 cmp r0, #2 beq loop2 b again loop1: mov r5,#16 ldr r1,=zero back1: ldrb r0, [r1] swi 0x200 ; Set 8 segment display to light up bl delay add r1,r1,#1 sub r5, r5,#1 cmp r5, #0 bne back1 b again loop2: mov r5,#16 ldr r1,=F back2: ldrb r0, [r1] swi 0x200 ; Set 8 segment display to light up bl delay sub r1, r1, #1 sub r5, r5,#1 cmp r5, #0 bne back2 b again delay: mov r4, #64000 loop3: sub r4, r4, #1 cmp r4, #0 bge loop3 mov pc, lr .data zero: .byte 0b11101101 one: .byte 0b01100000 two: .byte 0b01101110 three: .byte 0b11111010 four: .byte 0b00110011----01110011 five: .byte 0b10101011 six: .byte 0b10101111 seven: .byte 0b01110000 eight: .byte 0b11101111 nine: .byte 0b11100011 A: .byte 0b11100111 B: .byte 0b00101111 C: .byte 0b10001101 D: .byte 0b01101110 E: .byte 0b10001111 F: .byte 0b10000111 c. Move a string from LEFT to RIGHT on the LCD display panel. .text mov r0 , #30 ; r0 = x mov r1 , #7 ; r1 = y mov r7 , #0 ldr r8 , =num ldr r8 , [r8] ldr r2 , =str loop: swi 0x204 bl sum cmp r0 , #0 subne r0 , r0 , #1 swieq 0x11 b loop sum: cmp r7 , r8 addne r7 , r7 , #1 bne sum swi 0x206 ;Clear one line in the display on the LCD screen.r0-line no(y) mov r7 , #0 mov pc , lr .data str: .asciz "HELLO WORLD" num: .word 15000 Student Exercises: 1. Execute the following programs on ARMSIM – PLUG-INS. a. Display hexadecimal digits [0-9,A-F] on the 8 segment display. ; Program to display 0 to F and F-0 on the 8 segment display depending ; on the which black button is pressed .text .global _start begin: mov r0, #0 mov r2,#0 again: swi 0x202 ; check whether ; black button pressed or not cmp r0, #1 beq loop1 cmp r0, #2 beq loop2 b again loop1: mov r5,#16 ldr r1,=zero back1: ldrb r0, [r1] swi 0x200 ; Set 8 segment display to light up bl delay add r1,r1,#1 sub r5, r5,#1 cmp r5, #0 bne back1 b again loop2: mov r5,#16 ldr r1,=F back2: ldrb r0, [r1] swi 0x200 ; Set 8 segment display to light up bl delay sub r1, r1, #1 sub r5, r5,#1 cmp r5, #0 bne back2 b again delay: mov r4, #64000 loop3: sub r4, r4, #1 cmp r4, #0 bge loop3 mov pc, lr .data zero: .byte 0b11101101 one: .byte 0b01100000 two: .byte 0b01101110 three: .byte 0b11111010 four: .byte 0b00110011----01110011 five: .byte 0b10101011 six: .byte 0b10101111 seven: .byte 0b01110000 eight: .byte 0b11101111 nine: .byte 0b11100011 A: .byte 0b11100111 B: .byte 0b00101111 C: .byte 0b10001101 D: .byte 0b01101110 E: .byte 0b10001111 F: .byte 0b10000111 b. Move a string from RIGHT to LEFT on the LCD display panel. .text mov r0 , #0 ; r0 = x mov r1 , #7 ; r1 = y mov r7 , #0 ldr r8 , =num ldr r8 , [r8] ldr r2 , =str loop: swi 0x204 bl sum cmp r0 , #30 addne r0 , r0 , #1 swieq 0x11 b loop sum: cmp r7 , r8 addne r7 , r7 , #1 bne sum swi 0x206 ;Clear one line in the display on the LCD screen.r0-line no(y) mov r7 , #0 mov pc , lr .data str: .asciz "HELLO WORLD" num: .word 15000 |

MPCA-Laboratory/Assignment/Hands-on/Project